ULTRASONIC TESTING REPORT

| JOB DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Customer |  |  | Vedant Equips. Sales & Services Pvt Ltd., Khandala |  |  |  |  |  |  |  |  | Report No.: |  |  |  | NIIT/VESSPL/UT/25-26/29 |  |  |  |
| Client |  |  | MTA AUTOMOTIVE |  |  |  |  |  |  |  |  | Report Date: |  |  |  | 17/10/2025 |  |  |  |
| Project |  |  | VC-200412 |  |  |  |  |  |  |  |  | Inspection Date |  |  |  | 14/10/2025 |  |  |  |
| Reference Std. |  |  | ASME Sec V Article 4 / ASTM SA 609 / ASTM SA 435 / ASTM 578 / ASTM SA 388 / CUSTOM |  |  |  |  |  |  |  |  | Inspection Time |  |  |  | 02.00 PM to 05.00 PM |  |  |  |
| Acceptance Criteria |  |  | ASME SEC VIII Div. 1 Appendix 12 / ASTM SA 609 / ASTM SA 435 / ASTM 578 / ASTM SA 388 / CUSTOM |  |  |  |  |  |  |  |  | Material |  |  |  | IS 2062 E-250 BR |  |  |  |
| Stage of Inspection |  |  | After welding / As Casting / As Rolled/ As Forged |  |  |  |  |  |  |  |  | Thickness |  |  |  | 6,12 & 16 MM |  |  |  |
| Extent of Examination |  |  | 10% / 100% / To the maximum extent possible / Custom |  |  |  |  |  |  |  |  | Surface condition |  |  |  | Smooth |  |  |  |
| Type of Joint |  |  | Butt / Corner / T Joint / NA |  |  |  |  |  |  |  |  | Surface Temperature |  |  |  | Room Temperature |  |  |  |
| EQUIPMENT DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Equip. Type |  |  | Einstein-II DGS / USM 36 / Kappawave / USM 100 / Custom |  |  |  |  |  |  |  |  | Sr. no. |  |  |  |  |  | E4238-0215 |  |
| Make |  |  | Modsonic / Waygate / Kappawave / Custom |  |  |  |  |  |  |  |  | Calibration due |  |  |  |  |  | 18-12-2025 |  |
| Couplant |  |  | Water / Oil / Oil+ Grease/Starch |  |  |  |  |  |  |  |  | Basic Calibration block |  |  |  |  |  | IIW V1 / IIW V2 |  |
| SEARCH UNIT DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Search unit |  | Model |  |  | Angle |  |  | Sr. No. |  |  | Crystal Size |  |  | Wave Mode |  |  |  | Frequency |  |
|  |  | Modsonic |  |  | 45° |  |  | 539962015 |  |  | 8x9 mm / 20x22mm |  |  | Shear |  |  |  | 2 / 4 MHz |  |
|  |  | Modsonic |  |  | 60° |  |  | 539932103 |  |  | 8x9 mm / 20x22mm |  |  | Shear |  |  |  | 2 / 4 MHz |  |
|  |  | Modsonic |  |  | 70° |  |  | 181IAB |  |  | 8x9 mm / 20x22mm |  |  | Shear |  |  |  | 2 / 4 MHz |  |
|  |  | Modsonic |  |  | Normal / TR |  |  | 539932103 |  |  | Ø10 mm / Ø24 mm |  |  | Longitudinal |  |  |  | 2 / 4 MHz |  |
| TECHNIQUE DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Modsonic | 70° | 181IAB | 8x9 mm | Shear | 4 MHz |
| UT Method |  |  |  |  |  | Pulse Echo |  |  |  |  |  | Reference Calibration block |  |  |  |  |  | 19 mm / 38 mm / Job Itself / Custom |  |
| UT Calibration Method |  |  |  |  |  | By DAC Method / DGS Method / Backwall Reduction Method |  |  |  |  |  | Scanning dB |  |  |  |  |  | Ref dB+6dB |  |
| Scanning Sensitivity |  |  |  |  |  | Ø 2.5 mm SDH / Ø 3mm SDH / 1st BWE set @ 80% of FSH on Job / Custom |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Angle Probe calibration detail. |  |  |  |  |  | 0° |  |  | 45° |  |  |  |  |  | 60° |  | 70° |  |  |
| Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1st Point |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2nd Point |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3rd Point |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ref dB |  |  |  |  |  | 35 dB |  |  | 40 dB |  |  |  |  |  | 46.0dB |  | 49.0dB |  |  |
| OBSERVATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sr. No. | Job Description |  |  | Drg No. / Joint No. |  | Size |  |  |  | Quantity in Nos. |  |  | Evaluation |  |  |  |  |  | Remark |
| 1 | T1J1 |  |  | XX |  | 10X300 mm |  |  |  | 12 |  |  | No relevant Indication Found / Relevant Indication Found |  |  |  |  |  | Accepted / Not Accepted |
| 2 | B1J1 |  |  | XX |  | 18X300 mm |  |  |  | 12 |  |  | No relevant Indication Found / Relevant Indication Found |  |  |  |  |  | Accepted / Not Accepted |
| EXAMINED BY National Industrial Inspection And Training |  |  |  |  |  |  | CUSTOMER: XXXXXXXX |  |  |  |  |  |  |  | CLIENT / TPI: XXXXXXX |  |  |  |  |
| Name: Mr. Mayur Bankar |  |  |  |  |  |  | Name: |  |  |  |  |  |  |  | Name: |  |  |  |  |
| UT NDE Level II: |  |  |  |  |  |  | Designation: |  |  |  |  |  |  |  | Designation: |  |  |  |  |
| Signature: |  |  |  |  |  |  | Signature: |  |  |  |  |  |  |  | Signature: |  |  |  |  |
| I.D. No.: 115 |  |  |  |  |  |  | I.D. No.: |  |  |  |  |  |  |  | I.D. No.: |  |  |  |  |
| Date: 17/10/2025 |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  | Date: |  |  |  |  |