LIQUID PENETRANT TEST REPORT

| JOB DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Customer |  | GOURAV ENGINEERS, BARAMATI |  |  |  |  | Report No.: |  |  |  | NIIT/GE/PT/25-26/01 |  |
| Client |  | ALLNX RESINS INDIA PVT LTD. |  |  |  |  | Report Date: |  |  |  | 11/04/2025 |  |
| Project |  | OIL COOLER FOR GEAR BOX |  |  |  |  | Inspection Date |  |  |  | 11/04/2025 |  |
| Reference standard |  | ASME Sec. V, Article VI / ASTM E 165/ Custom |  |  |  |  | Inspection Time |  |  |  | 10.00 AM – 03.30 PM |  |
| Acceptance Criteria |  | ASME Sec. VIII Div. I, Appendix 7 / Appendix 8 / Custom |  |  |  |  | Material |  |  |  | IS 2062 E250 BR |  |
| Stage of Inspection |  | After welding / As Casting / After Machining |  |  |  |  | Thickness |  |  |  | 6,12 & 16 MM |  |
| Extent of Examination |  | 10% / 100% / To the maximum extent possible / Custom |  |  |  |  | Surface condition |  |  |  | Smooth |  |
| Type of Joint |  | Butt / Corner / T Joint / NA |  |  |  |  | Surface Temperature |  |  |  | Room Temperature |  |
|  |  |  |  |  |  |  | Welding Process |  |  |  | SMAW/ GMAW/ FCAW/ SAW/ GTAW/ MAG |  |
| METHOD DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |
| Penetrant Method |  | Visible Method / Fluorescent / Dual |  |  |  |  |  |  |  |  |  |  |
| Excess Penetrant Removal method |  | Solvent Removal / Water Washable / Post Emulsifiable |  |  |  |  |  |  |  |  |  |  |
| CONSUMABLES DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |
| Material |  | Manufacture |  |  |  |  |  |  | Batch |  | Expiry Date |  |
| Penetrant |  | Pradeep / Dyeglo / Ferrochem / MR Chem / Magnaflux / Custom |  |  |  |  |  |  | 1982 |  | 08/26 |  |
| Developer |  | Pradeep / Dyeglo / Ferrochem / MR Chem / Magnaflux / Custom |  |  |  |  |  |  | 8397 |  | 10/25 |  |
| Cleaner |  | Pradeep / Dyeglo / Ferrochem / MR Chem / Magnaflux / Custom |  |  |  |  |  |  | 8337 |  | 10/25 |  |
| METHOD DISCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |
| Dwell time |  | 7 to 10 Min |  |  |  |  |  |  | Light Intensity |  | 1250 lux |  |
| Developing Time |  | 15-20 Min |  |  |  |  |  |  | Light Equip. Used |  | 60 W Bulb / NA |  |
| Post Cleaning |  | Done |  |  |  |  |  | Drying Time |  |  | 10 Min / Custom / NA |  |
| OBSERVATIONS |  |  |  |  |  |  |  |  |  |  |  |  |
| Sr. No. | Job Description |  | Drg No. / Joint No. | Size |  | Quantity in Nos. | Evaluation |  |  |  |  | Remark |
| 1 | T1J1 |  | XX | 10X300 mm |  | 12 | No relevant Indication Found / Relevant Indication Found |  |  |  |  | Accepted / Not Accepted |
| 2 | B1J1 |  | XX | 18X300 mm |  | 12 | No relevant Indication Found / Relevant Indication Found |  |  |  |  | Accepted / Not Accepted |
| EXAMINED BY National Industrial Inspection And Training |  |  |  |  | CUSTOMER: XXXXXXXX |  |  |  |  | CLIENT / TPI: XXXXXXX |  |  |
| Name: Mr. Mayur Bankar |  |  |  |  | Name: |  |  |  |  | Name: |  |  |
| PT NDE Level II: |  |  |  |  | Designation: |  |  |  |  | Designation: |  |  |
| Signature: |  |  |  |  | Signature: |  |  |  |  | Signature: |  |  |
| I.D. No.: 115 |  |  |  |  | I.D. No.: |  |  |  |  | I.D. No.: |  |  |
| Date: 17/10/2025 |  |  |  |  | Date: |  |  |  |  | Date: |  |  |